TECHNICAL SPECIFICATIONS AND TECHNICAL TENDER

Contracting Authority: FEROPLAST Ltd., Grožnjanska ulica 5, 52460 Buje, Republic of Croatia

Subject of procurement: Procurement of machinery

Publication reference: 5/2019

TECHNICAL TENDER

Tenderers must fill in the column 2 and enter detailed description of the offered goods (words like “compliant” or “yes” are not sufficient). Tenderer must not change the specifications indicated in the column number 1.

For all producers, types of products, standards or norms indicated in technical specifications shall apply „or equivalent“. Eventual accompanying documentation which the Tenderer submits as the supplement to the tender must clearly indicate the models, or options which are offered. Tenders which do not identify precisely models and specifications may be rejected.

Note: Tenderer fills technical specifications only for the group or groups for which the offer is submitted!

TEHNICAL SPECIFICATIONS

| I Requiered specifications |  | II Offered specifications |
| --- | --- | --- |
| GROUP 1 |  |  |
| 1. Straightening and cutting line machine Piece: 1 |  |  |
| Parametar / item | Characteristics |  |
| Entry material - wire | Ribbed or smooth wire Ø 2 - 7 mm; Spool |  |
| Maximum spools weight | maximum to 1.200 kg |  |
| Wire direction | wire path from left to right |  |
| Raw material | Spools Ø2,00 - 7,00 mm; UTS (Ultimate tensile strength) until 750 Mpa |  |
| Wire feed speed | 20 - 120 m/min |  |
| Minimum cutting length | minimum 100 mm |  |
| Maximum cutting length | maximum to 5.000 mm |  |
| Flatness tolerance | from 1mm on 2.000 mm |  |
| Maximum length tolerance | ± 0.2 mm |  |
| Unwinding the wire | automatic |  |
| Additional requests | - Installation documentation - instructions for safe operation - maintenance documentation - (all in digital format) |  |
| Additional documentation | - Electro scheme - Pneumatic scheme - Mechanical parts scheme - Spare parts list |  |
| GROUP 2 |  |  |
| 1. Wire butt welding machine Piece: 1 |  |  |
| Parametar / item | Characteristics |  |
| Power supply | 400 V / 50 Hz |  |
| Minimum rated power at 50% ED | minimum 25 kVA |  |
| Minimum clamping force | minimum 900 daN |  |
| Minimum squeeze force | minimum 350 daN |  |
| Additional requests | - Installation documentation - instructions for safe operation - maintenance documentation - (all in digital format) |  |
| Additional documentation | - Electro scheme - Pneumatic scheme - Mechanical parts scheme - Spare parts list |  |
| GROUP 3 |  |  |
| 1. Machine for electro resistance welding Piece: 1 |  |  |
| Parametar / item | Characteristics |  |
| Projection plates throat depth | minimum 650 mm |  |
| Minimum nominal power | minimum 150 kVA |  |
| Electrode stroke | minimum 100 mm |  |
| Additional requests | - Installation documentation - instructions for safe operation - maintenance documentation - (all in digital format) |  |
| Additional documentation | - Electro scheme - Pneumatic scheme - Mechanical parts scheme - Spare parts list |  |
| GROUP 4 |  |  |
| 1. Salt spray test chamber Piece: 1 |  |  |
| Parametar / item | Characteristics |  |
| Salt chamber type | Possibility to check according ISO 9227 and ISO 6270-2CH or equivalent |  |
| Capacity | minimum 400 liters |  |
| Inner dimension | minimum 1.000 x 600 x 1.000 mm |  |
| Wetting and drying | from 12°C to 50°C |  |
| Salt reservoir | minimum 80 liters |  |
| Stand for samples - spiked suspension | minimum 6 pieces |  |
| Certified salt for probes | minimum 100 kg |  |
| Prague loading | 800 mm |  |
| Additional requests | - Noise level – less than 50 dB - Calibration certifyicate or equivalent |  |
| Additional documentation | - Installation documentation - instructions for safe operation - maintenance documentation - (all in digital format) |  |
| GROUP 5 |  |  |
| 1. Forklift Piece: 1 |  |  |
| Parametar / item | Characteristics |  |
| Main power | LPG (Liquified petroleum gas) |  |
| Manipulation | From the seat |  |
| Tires | filled |  |
| Height of the lowered crane | maximum to 2.300 mm |  |
| Free moving fork | minimum 150 mm |  |
| Lift height | 3.200 mm or more |  |
| Maximum lifting at the highest lift height | minimum 1.700 kg |  |