TECHNICAL SPECIFICATIONS

CONTRACTING AUTHORITY: Reta d.o.o., Zagrebačka 15 K, 47000 Karlovac, Croatia

PIN: 01201898816

SUBJECT OF PROCUREMENT: Wire drawing lines with training of employees

TECHNICAL OFFER

Tenderers are required to fill in column 3 with a detailed description of the goods offered (words such as "compliant" or "yes" are not sufficient). The Tenderer shall not change the specifications stated in column 2.

For all manufacturers, types of products, standards or norms that are set out in the technical specifications applies "or equivalent”. Any supporting documentation which the Tenderer submits to complement the offer shall clearly indicate the models or options being offered. Offers which do not identify precisely the models and specifications may be rejected.

Technical specifications listed in the table in the check-list format represent minimum standard for every single item of the requested specifications and are sole basis for evaluating the technical compliance of the offers. The Tenderer can also offer products that meet higher standards or technical specifications better than the minimum.

| I Nr. Item | II Requested specification |  |  | III Offered specification |
| --- | --- | --- | --- | --- |
| 1. | Line for drawing high carbon wire, 11 blocks – 1 piece |  |  |  |
|  |  | Quantity: |  | 1 |
|  |  | Manufacturer Name: |  |  |
|  |  | Model Name: |  |  |
|  | Element of line | Parameter/item | Characteristics (minimum requirements) |  |
| 1.1. | Wire Drawing Line | Material | Wire carbon content min. 0,3% |  |
|  |  | Rod preparation | Chemically picked, mechanically descaled, phospate |  |
|  |  | Final product | High carbon wire |  |
|  |  | Rod range on descaling unit/pay-off unit | Diameter 5,5 mm |  |
|  |  | Tensile strength | Max. inlet 1250 N/ mm2, Max. outlet 2100 N/ mm2 |  |
|  |  | Wire range (HC) | Min. 1,40 mm |  |
|  |  | Wire direction | From right to left |  |
| 1.2. | Twin rod Pay-Off | Machine speed | Max. 3,5 m/sec |  |
|  |  | Design | For high carbon wire rods |  |
|  |  | Rod inlet | 5,5 mm |  |
|  |  | Hydraulic unit | Power min. 4,0 kW |  |
| 1.3. | Reverse bender | Rollers | 2 units of 3 operating rollers each Entrance guiding rollers, 1 piece Exit guiding rollers, 1 piece |  |
|  |  | Hydraulic pump | Power min. 0,55 kw, 2 hydraulic cylinders |  |
| 1.4. | Patented Double head sand belt cleaner | Heads | 2 piece |  |
|  |  | Linear speed | Max 6 m/sec |  |
|  |  | Drives and motors | AC motors |  |
|  |  | Dust extraction | Single dust extraction tube, dust cartridge filter |  |
| 1.5. | Rinsing Unit | Filter Tank capacity | Min. 0,15 m3 |  |
|  |  | Material | Stainless steel |  |
|  |  | Water filter unit | With a combined magnetic disc unit and paper filter |  |
| 1.6. | Salt Coating Unit | Heaters | 4 pieces, Temperature digital display |  |
|  |  | Installed Power | Min. 15 kW |  |
|  |  | Solution tank | Min. volume 0,3 m3 |  |
|  |  | Working solution | Min. 85 °C |  |
|  |  | Additional heaters | 1 piece |  |
|  |  | Isolation walls | Min. 60 mm thickness to reduce heat dispersion |  |
| 1.7. | Dryer | Heaters | 2 pieces with air temperature control |  |
|  |  | Isolation | Fiber wool |  |
| 1.8. | Wire Drawing Machine | Number of blocks | 11 pieces |  |
|  |  | Number of die boxs | 11 pieces |  |
|  |  | Cooling | With water |  |
|  |  | Final block Double Deck type | With one set of wire straighteners |  |
|  |  | Equipment | Die equipmet, 11 pieces; Tungsten carbide on sheaves, 1 piece Tungsten carbide on the intermediate sensor rollers, 10 pieces Tungsten carbide on input rollers and output sheave (take-up pairing), 1 piece Tungsten carbide on input and output rollers (spooler pairing), 1 piece Tungsten carbide on the final dancer arm, 1 piece Tungsten carbide on the pulling band of the capstans, 11 pieces Flow switch, 1 piece Rotating die box, 2 piece Soap Stirrer, 11 pieces Antirust coating inside the, 11 pieces Vibration dumping rollers, 7 pieces Electrical plug, 3 pieces Water flow control lights, 11 pieces |  |
| 1.9. | Horizontal Drawing Coiler | Block dimensions | Diameter 600 mm |  |
|  |  | Colling block | Water and air |  |
|  |  | Patterning | Elevator type |  |
|  |  | Tungsten carbide | On sheaves and on the pulling band of the capstans |  |
| 1.10. | Vertical Spooler | Adjustable ramps | Separate and following drawing machine ramps |  |
|  |  | Tungsten carbide | On traverse sheaves |  |
| 1.11. | Automation Drives and Controls | Control cabinet | Single side cubicle , main power supply cables to the control cabinet entering from the top |  |
|  |  | Cooling | Fan and filter |  |
|  |  | Configuration | Main circuit magnetic breaker, Remote PLC system programing with external and internal components as drive, video keyboard Web Assistance and IoT hardware device Operator panel fit in a suitable position close to the drawing machine, with all push buttons and lamps needed for the easy and correct use of the line supplied with a LCD Display, keyboard, as Man Machine Interface. |  |
|  |  | Reading system on display | Metric |  |
| 2. | Line for drawing low carbon wire, 5 blocks – 1 piece |  |  |  |
|  |  | Quantity: |  | 1 |
|  |  | Manufacturer Name: |  |  |
|  |  | Model Name: |  |  |
|  | Element of line | Parameter/item | Characteristics (minimum requirements) |  |
| 2.1. | Wire Drawing Line | Material | Wire carbon content max. 0,3% |  |
|  |  | Rod preparation | Mechanically descaled |  |
|  |  | Final product | Low carbon wire |  |
|  |  | Rod range on descaling unit/pay-off unit | Diameter 2,0 – 3,0 mm |  |
|  |  | Tensile strength | Max inlet 800 N/mm2, Max. Outlet 1000 N/mm2 |  |
|  |  | Wire direction | From left to right |  |
| 2.2. | Motorised spool type pay-off on vertical axis | Housing | Made is of welded construction and stress relieved |  |
|  |  | Spool | Motor drive |  |
| 2.3. | Wire Drawing Machine | Blocks | 5 pieces, diameter 300 mm |  |
|  |  | Die boxes | 5 pieces |  |
|  |  | Cooling | With water |  |
|  |  | Equipment | Die equipmet, 5 pieces; Tungsten carbide on the intermediate sensor rollers, 4 pieces Tungsten carbide on input rollers and output sheave (take-up pairing), 1 piece Tungsten carbide on the final dancer arm, 1 piece Tungsten carbide on the pulling band of the capstans, 5 pieces Flow switch, 1 piece Rotating die box, 1 piece Soap Stirrer, 5 pieces Antirust coating inside the, 5 pieces Vibration dumping rollers, 5 pieces Electrical plug, 2 pieces Water flow control lights, 5 pieces |  |
| 2.4. | Horizontal Static Coiler | Block dimensions | Diameter 300 mm |  |
|  |  | Cooling | Air |  |
|  |  | Tungsten carbide | On sheaves and on the pulling band of the capstans |  |
|  |  | Small coils swinging carrier | 4 positions |  |
| 2.5. | Automation Drives and Controls | Control cabinet | Single side cubicle , main power supply cables to the control cabinet entering from the top |  |
|  |  | Cooling | Fan and filter |  |
|  |  | Configuration | Main circuit magnetic breaker, Remote PLC system programing with external and internal components as drive, video keyboard, Web Assistance and IoT hardware device Operator panel fit in a suitable position close to the drawing machine, with all push buttons and lamps needed for the easy and correct use of the line supplied with a LCD Display, keyboard, as Man Machine Interface. |  |
|  |  | Reading system on display | Metric |  |
| 3. | Training of employees |  | Training of employees for the use of new technological solutions at the location of the Contracting Authority. Min. 20 days training for six employees. |  |
| 4. | Warranty period |  | Min. 12 months from the date of the shipping of goods |  |
| 5. | Delivery |  | Place of delivery of the subject of procurement is: RETA d.o.o., Zagrebačka 15K, 47000 Karlovac, Croatia. Provided delivery must be minimum CIP Karlovac Croatia, Incoterms. |  |