NARUČITELJ / CONTRACTING AUTHORITY: ADORO d.o.o. za proizvodnju, Kanalska ulica 36, 10 250 Zagreb , Hrvatska, OIB: 98931889409

PREDMET NABAVE / SUBJECT OF PROCUREMENT: Savijačica profila s edukacijom / Profile bending machine with education

Kolone I-II ispunjava Naručitelj – već su popunjene od strane Naručitelja

Kolonu III ispunjava Ponuditelj

Ponuđači su dužni ispuniti predloške na sljedećim stranicama:

- Kolona II je ispunjena od strane naručitelja i prikazuje tražene tehničke specifikacije (nije dozvoljena modifikacija istih od strane ponuđača),

- Kolonu III ispunjava ponuditelj sa detaljima ponuđene robe i usluge (npr. samo riječi “da” ili “ne” nisu dovoljne) – obvezna kolona. Ona ponuditelju omogućava unos komentara na svoju predloženu robu.

- Ponuda mora biti dovoljno jasna kako bi Odbor za ocjenu ponuda mogao napraviti jednostavnu usporedbu između traženih i ponuđenih specifikacija.

Nabava: SAVIJAČICA PROFILA S EDUKACIJOM

Napomena: Za sve norme, tipove, robne marke i modele navedene u tehničkim specifikacijama primjenjuje se „ili jednakovrijedno“.

Columns I-II are filled out by Contracting Authority - already filled out by Contracting Authority

Column III is filled out by the Tenderer

Tenderers must fill out the form with the following columns:

- Column II is filled out by the Contracting Authority and indicates all required technical specifications (the Tenderer is not allowed to modify technical specifications),

- Column III is filled out by the Tenderer with details of offered goods and services (e.g. only words like “yes” or “no” are not sufficient) – obligatory column. It enables the Tenderer to enter comments to all goods offered.

- The Tender must be understandable for Members of the Board to evaluate the Tender and make the simple comparison between the required and offered technical specifications.

Procurement of Profile bending machine with education

Note: For all norms, types and trade marks, models indicated in technical specification the term „or equivalent“ is applicable.

TEHNIČKE SPECIFIKACIJE / TECHNICAL SPECIFICATIONS

| Redni broj/ Ord.No. | Tražene tehničke specifikacije / Requested technical and functional specifications | Ponuđene tehničke specifikacije / Offered technical specifications |
| --- | --- | --- |
| 1. | Valjci za savijanje obradka Minimalno 2 kom / Rolls for bending workpieces Minimum 2 pcs |  |
| 1.1. | Svi valjci moraju se kretati pojedinačno / All rolls must be driven individually |  |
| 1.2. | - Kontinuirana brzina: minimum 0-20 roll/min / Continuous speed: minimum 0-20 roll/min - Podesivi razmak između prednjih valjaka / Adjustable distance between front rollers - Snaga pritiska po x-osi: min. 15 t / Pressure power on x-axis: min. 15 t - Tolerancija pozicioniranja x-osi: 0.01 mm / Accuracy and positioning x-axis: 0.01 mm |  |
| 2. | Upravljanje računalom s dodirnim zaslonom / Touch screen computer control |  |
| 3. | Program za upravljanje radom stroja mora imati: - Prilikom izravnog kreiranja programa savijanja, sustav mora provjeriti svaki unos i time vidjeti je li moguć izračun, ukazujući na pogreške gdje je to prikladno. - Izravan nadzor s grafičkim prikazom - Koristeći unešene pojedinosti, program mora izračunati cijeli programski slijed s relevantnim prilagodbama, prijelazima, itd. Ukoliko se tijekom savijanja pojave odstupanja, efektivni polumjer savijanja programu mora se moći dati odrediti od strane operatera, a vrijednost korekcije automatski izračunati iz toga, a operacija prilagoditi potrebi. Program for managing the operations of machine must have: - When creating a bending program directly, the system must check each entry to see whether a calculation is possible, indicating errors where appropriate. - Direct monitoring with graphical display - Using the entered details, the program must calculate the entire program sequence with the relevant adjustments, transitions, etc. Where variances arise during bending, the effective bend radius must can be specified for the program by operater, and a correction value must be automatically calculated from this and the operation must be adjusted as appropriate. |  |
| 4. | Ažuriranje i daljinska dijagnostika putem internet modema / Up-date and remote diagnostics via Internet modem. |  |
| 5. | Specijalni kombinirani alat za savijanje profila prema Tehničkom nacrtu Naručitelja / Special combination tool for bending profiles according to Tehnical drawing of Contracting Authority |  |
| 6. | Edukacija 3 zaposlenika Naručitelja / Education of 3 employees of Contracting Authority |  |
| 7. | Instalacija na adresi Naručitelja / Installation at the address of Contracting Authority |  |
| 8. | Jamstvo: minimalno 12 mjeseci / Warranty: minimally 12 months |  |

Mjesto i datum (Place and date)								za Ponuditelja (for the Tenderer)


________________________										__________________________													potpis ovlaštene osobe /(signature of the autorized person)