ANNEX 9 - TECHNICAL REQUIREMENTS

Procurement registration number: DILJ-MF-01-22 / NOJN-02

PROJECT:

Energy efficiency improvement and generation of electricity from renewable sources of the Dilj production plants

ANNEX 9

TECHNICAL REQUIREMENTS

1 . Technical characteristics of the plant

1.1 Power supply

It is envisioned to use the power supply of existing plants to be replaced. Power supplies are 3x400V AC / 50Hz with grounding.

1.2 Compressed air supply

It is planned to connect to the existing compressed air supply system in the factory, which is a minimum of 6 Bar, with an air drying system.

1.3 Earthing installation

Protection against hazardous contact voltages is provided by automatically switching off the power supply in the TN-CS system, which is used in the factory. Use the yellow-green core of each cable only for protective earth (PE).

Equalization of potentials of conductive parts of machines should be performed with Fe / Zn tape and connected to already existing earthing installations on the site, ie connected to the bus for equipotential bonding in distribution cabinets.

1.4 Plant characteristics

PRODUCTION DATA

Setting, unsetting and packaging plants operate in 2 shifts, 300 days a year. The working time of one shift is 8 hours, of which 6.5 hours is effective work.

Products and their technical characteristics are given in the following table:

| TYPE | DIMENSIONS [mm] |  |  | WEIGHT [Kg] |  | NUMBER OF PIECES ON DRYER CAR |
| --- | --- | --- | --- | --- | --- | --- |
|  | WIDTH | HEIGHT | LENGTH | DRY | FIRED |  |
| MB2 Optim | 190 | 190 | 250 | 6.8 | 6.3 | 240/288 |
| MB3 Optim | 190 | 190 | 290 | 7.9 | 7.3 | 240 |
| TB20 Optim | 200 | 238 | 380 | 15 | 13.8 | 160 |
| TB25 Optim | 250 | 238 | 380 | 18.4 | 17 | 160 |
| TB30 Optim | 300 | 238 | 300 | 14.9 | 13.8 | 200 |
| TB38 Optim | 380 | 238 | 250 | 18.8 | 17.2 | 160 |
| SB14 | 428 | 140 | 240 | 9.7 | 9 | 120 |
| PB-50/10/19 | 100 | 190 | 500 | 9.6 | 8.8 | 144 |
| PB-50/12/19 | 120 | 190 | 500 | 10.7 | 9.8 | 144 |
| PB-50/10 / 23.8 | 100 | 238 | 500 | 12 | 11 | 120 |
| PB-50/12 / 23.8 | 120 | 238 | 500 | 13.4 | 12.3 | 120 |

Table 1. Technical characteristics of products

SETTING OF DRY BLOCK FROM DRYER CAR TO KILN CAR

Figure 1. Existing line for setting of dry goods

The existing dry goods setting facility consists of:

- Unloading of the dryer car with transport of the dryer car, elevator and dry block pusher from the dryer car

- Transport of dry block with preparation of setting for kiln car

- Handling manipulator with grippers

- Hydraulic pusher of the kiln car under the setting manipulator

- Chain transport of kiln cars before loading place (transport 4)

- Chain transport of kiln cars after loading place (transport 5)

The project envisions the retention of the elevator for unloading the dryer car and its connection and integration into the control system of the new plant. The chain transport of the dryer car in front of the unloading elevator is retained, as well as the transport of the kiln car before and after the setting position. The new control system must directly receive sensor signals and control the actuators of the equipment that is being retained.

List of motors of equipment to be retained:

| # | POSITION | MANUFACTURER | MODEL | POWER | COS ɸ | SPEED | BRAKE |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Chain before unloading elevator | MGM | BA90 LA6 | 1.1 kW | 0.68 | 910 | 400V |
| 2 | Unloading elevator drive | MGM | BAF 132 MA4 | 7.5 kW | 0.84 | 1435 | 400V |
| 3 | Transport chain for VTP before reloading | SEVER | KZK100Ld8 | 1.1 kW | 0.73 | 680 |  |
| 4 | Transport chain for VTP after reloading | SEVER | KZA112M8 | 1.5 kW | 0.71 | 700 |  |

Table 2.  Motors of setting line equipment to be retained

The dryer unloading elevator consists of the following sensors:

| # | FUNCTION | TYPE |
| --- | --- | --- |
| 1 | CAR IN THE UP POSITION | MECH. END SWITCH |
| 2 | CAR IN DOWN POSITION | MECH. END SWITCH |
| 3 | SAFETY – COUNTERWEIGHT IN UPPER POS. - LEFT | MECH. END SWITCH |
| 4 | SAFETY – COUNTERWEIGHT IN UPPER POS. - RIGHT | MECH. END SWITCH |
| 5 | ELEVATOR LINED UP WITH RAIL - STOP | MECH. END SWITCH |
| 6 | ELEVATOR LINED UP WITH RAIL - PUSHING CONDITION | MECH. END SWITCH |
| 7 | ELEVATOR TRAVELLING UP – LAYER STOP | MECH. END SWITCH |
| 8 | ELEVATOR TRAVELLING DOWN – LAYER STOP | MECH. END SWITCH |
| 9 | SAFETY - LIFT IN UPPER POSITION | MECH. END SWITCH |
| 10 | ENDSTOP – LIFT IN UPPER POSITION | MECH. END SWITCH |

Table 3. Sensors of unloading elevator

The rest of the retained setting line equipment consists of the following sensors:

| # | FUNCTION | TYPE |
| --- | --- | --- |
| 1 | Chain before unloading elevator - chain back endstop | MECH. END SWITCH |
| 2 | Chain before unloading elevator - chain forward endstop | MECH. END SWITCH |
| 3 | Chain before unloading elevator - elevator operation permit | MECH. END SWITCH |
| 4 | Dryer car in front of the unloading elevator | INDUCTIVE SWITCH |
| 5 | Transport chain for kiln car before setting position – back endstop | MECH. END SWITCH |
| 6 | Transport chain for kiln car before setting position – forward endstop | MECH. END SWITCH |
| 7 | Kiln car in front of the handling machine | PHOTO SWITCH |
| 8 | Transport chain for kiln car after setting position - back endstop | MECH. END SWITCH |
| 9 | Transport chain for kiln car after setting position – forward endstop | MECH. END SWITCH |
| 10 | Kiln car at the exit of the handling machine | MECH. END SWITCH |

Table 4. Sensors of setting line equipment to be retained

The new plant should include a new pusher of dryer car layers for elevator, a new dry block transport with the ability to rotate/tilt the blocks and a programming table for preparation of blocks for loading onto the kiln car, and a new robotic manipulator for loading the dry blocks onto the kiln cars.

The previously mentioned parts of the dryer and kiln car transport equipment needs to be retained and operated with the control system of the new setting line. The existing hydraulic kiln car pusher below the setting position needs to be replaced with a new car positioning system and a new kiln car locking system needs to be installed for precise positioning of the  kiln car.

Communication connection with other equipment and machines (digital signals) needs to be maintained and integrated into the new control system.

The new plant control cabinet must have at least one additional three-phase power distribution (fuse base and output terminals) to connect the external load.

UNSETTING OF THE FIRED BLOCK AND PACKAGING

Figure 2. Existing unsetting and packing line

The existing plant for unsetting the fired block and packaging consists of:

- Manipulator for unloading of kiln cars with grippers

- Chain pallet transport (5)

- OMS AT53 pallet thermohooding machine

- The hydraulic pusher of the kiln car under the unloading manipulator

- Chain-rope transport of kiln cars (transport 1)

- Chain transport of kiln cars before unloading position (transport 2)

- Chain transport of kiln cars after unloading position (transport 3)

The project envisions the retention of the OMS AT53 thermohooding machine, the pallet transport system (to the extent appropriate for the new plant), the transport of kiln cars and the management of these parts by the control system of the new plant. The new control system must directly receive sensor signals and control the actuators of the unsetting line equipment that is retained, except for the OMS AT53 thermohooding machine for which the signals for communicating with the new setting line control system are given in Table 7. The existing hydraulic kiln car pusher below the unloading position needs to be replaced with a new car positioning system and a new kiln car locking system needs to be installed for precise positioning of the kiln car.

List of motors for equipment that can be retained:

| # | POSITION | MANUFACTURER | MODEL | POWER | COS ɸ | SPEED |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Transport chain-rope for kiln cars before unloading position | KONČAR | 5AZ132S-8 | 2.2 kW | 0.75 | 690 |
| 2 | Transport chain for kiln cars before unloading position | ? |  | 2.2 kW |  |  |
| 3 | Transport chain for kiln cars after unloading position | SEVER | KPC 112 M8 | 1.5 kW | 0.71 | 700 |
| 4 | Pallet conveyor chain 1 | MGM | BAPV 90LA2 | 2.2 kW | 0.88 | 2840 |
| 5 | Pallet conveyor chain 2 | SEVER | KZK 90 S4 | 1.1 kW | 0.80 | 1400 |
| 6 | Pallet conveyor chain 3 | KONČAR | 5AZ100LB-8 | 1.1 kW | 0.65 | 690 |
| 7 | Pallet conveyor chain 4 | SEVER | KZK 90 S4 | 1.1 kW | 0.80 | 1400 |
| 8 | Pallet conveyor chain 5 | SEVER | SK 100 L / 40 | 3.0 kW | 0.80 | 1415 |

Table 5. Motors of unsetting line that can be retained

The rest of the unsetting line equipment that can be retained consists of the following sensors:

| # | FUNCTION | TYPE |
| --- | --- | --- |
| 1 | Transport rope before unloading pos.- kiln car in position |  |
| 2 | Transport chain before unloading pos.- back endstop | MECH. END SWITCH |
| 3 | Transport chain before unloading pos.- front endstop | MECH. END SWITCH |
| 4 | Transport chain after unloading pos.- back endstop | MECH. END SWITCH |
| 5 | Transport chain after unloading pos.- forward endstop | MECH. END SWITCH |
| 6 | Kiln car in front of the unloading machine |  |
| 7 | Kiln car at the exit of the unloading machine |  |
| 8 | Pallet transport chain 1 – pallet detection |  |
| 9 | Pallet conveyor chain 2 – pallet in position |  |
| 10 | Pallet transport chain 3 – pallet on a chain | PHOTO SWITCH |
| 11 | Pallet conveyor chain 4 – pallet at the entrance to the thermofoil machine | PHOTO SWITCH |
| 12 | Pallet conveyor chain 4 – pallet in foil position | PHOTO SWITCH |
| 13 | Pallet conveyor chain 4 – stop foil pallets | PHOTO SWITCH |
| 14 | Pallet conveyor chain 4 – pallet at the exit of the foil | PHOTO SWITCH |
| 15 | Pallet transport chain 5 – pallet stacker | PHOTO SWITCH |
| 16 | Pallet conveyor chain 5 – stop pallet on a chain | PHOTO SWITCH |
| 17 | Pallet transport chain 5 – safety stop pallet on the chain | PHOTO SWITCH |

Table 6. Sensors of unsetting line equipment that can be retained

The OMS AT53 thermohooding machine must be connected to the new control system with a minimum of the following signals:

| # | FUNCTION | TYPE |
| --- | --- | --- |
| 1 | OMS end of cycle | INPUT (BIT) |
| 2 | OMS out of package dimensions | INPUT (BIT) |
| 3 | OMS security | INPUT (BIT) |
| 4 | OMS machine blocked | INPUT (BIT) |
| 5 | OMS start cycle | OUTPUT (BIT) |
| 6 | OMS start sack creation | OUTPUT (BIT) |

Table 7. Signals of OMS AT53 thermohooding machine

The new plant should include a new robotic manipulator for unloading the kiln car, a new transport of the fired block with programming/preparation for stacking on pallets with the possibility of rotating/tilting of blocks, a new robotic manipulator for stacking and forming pallets of the fired block.

The retained transport of kiln cars and the transport of pallets need to be integrated into the new control system.

The existing communication link with other plants (digital signals) needs to be maintained and integrated into the new control system.

The new control cabinet of the unloading plant must retain the three-phase power distribution for the control cabinet of the OMS machine, the power supply of the central vacuum cleaner and the power supply to the output door of the dryer.

2 . Requirements for a new lines

The project should include the following parts:

- The dry block setting line consists of:

  - Pusher for dry block rows of dryer car

  - Transport of the dry block with programming and possible rotation/tilting of the block for preparation for stacking on the kiln car

  - Robotic manipulator for stacking the block on the kiln car

  - Robotic manipulator gripper

  - Robot manipulator support base (documentation for local construction)

  - Control system for the entire equipment and other components that are retained, as well as the necessary sensors, cables, mechanical, electrical, pneumatic and hydraulic components.

  - The positioning, centering, stopping and locking system of the kiln car at the loading position

  - Plant safety equipment (doors, locks and / or light barriers to restrict access) and documentation for local fence construction

  - Integration of other parts of the plant that are retained

  - Clearly defined position for removing the breakage of the dry block and the position of throwing the breakage into the waste box or on the rubber conveyor belt (box and/or belt will be provided by the Ordering party)

  - Clearly defined position of the control part of the plant with the position of the line operator

- The plant for unloading and packing the fired block consists of:

  - Robotic manipulator for unloading kiln cars

  - Robotic manipulator gripper

  - Robotic manipulator support base (documentation for local construction)

  - Transport of the fired block with programming and possible rotation/tilting of the block for preparation for stacking on a pallet

  - Storage and dispenser of wooden pallets with necessary transport

  - Robotic manipulator for stacking and forming a pallet of fired blocks

  - Robotic manipulator gripper

  - Robot manipulator support base (documentation for local stand construction)

  - Storage for thin polystyrene sheets in reach of robot for stacking and forming of pallets so it could individually pick them up with gripper and insert them between rows of block on a pallet.

  - Control system for the entire equipment and other components that are retained, as well as the necessary sensors, cables (excluding power supply, and cables for connecting old equipment that will be retained), mechanical, electrical, pneumatic and hydraulic components.

  - The positioning, centering, stopping and locking system of the kiln car at the unloading position

  - Plant safety equipment (doors, locks and / or light barriers to restrict access) and documentation for local fence construction

  - Integration of other parts of the plant that are retained

  - Bus (ProfiNET) connection between setting and unloading line.

  - Clearly defined position for removing the breakage of the fired block and the position of throwing the breakage into the waste box or on the rubber conveyor belt (box and/or belt will be provided by the Ordering party)

  - Clearly defined position of the control part of the plant with the position of the line operator

- Development of control programs and user interfaces of entire plants in Croatian or English with a visual display of all activated sensors, actuators and alarms

- Preparation of documentation of entire plants in Croatian or English

- Management and supervision of the installation of the plant with the help of employees of the Ordering party’s company

- Commissioning, training of employees of the Ordering party’s company and proving the operating parameters

The minimum capacities of the new plants are given in the following table of products with regard to the setting configurations from the Draft of setting configurations (Annex 8) in tons of fired goods on a working day, according to the given schedule:

| TYPE | SETTING CONFIGURATION ACCORDING TO ANNEX 8. | NUMBER OF SHIFTS PER DAY | EFFECTIVE WORKING HOURS PER SHIFT | MINIMUM CAPACITY |
| --- | --- | --- | --- | --- |
|  |  |  | [h] | [T / day] |
| MB2 Optim * | Standard | 2 | 6.5 | 360 |
| MB2 Optim * | B | 2 | 6.5 | 360 |
| MB3 Optim | Standard | 2 | 6.5 | 360 |
| MB3 Optim | A | 2 | 6.5 | 360 |
| MB3 Optim | B | 2 | 6.5 | 360 |
| TB20 Optim | Standard | 2 | 6.5 | 380 |
| TB25 Optim | Standard | 2 | 6.5 | 380 |
| TB30 Optim | Standard | 2 | 6.5 | 380 |
| TB38 Optim | Standard | 2 | 6.5 | 380 |
| SB14 | Standard | 2 | 6.5 | 200 |
| SB14 | A | 2 | 6.5 | 200 |
| PB-50/10/19 | Standard | 2 | 6.5 | 220 |
| PB-50/12/19 | Standard | 2 | 6.5 | 220 |
| PB-50/10 / 23.8 | Standard | 2 | 6.5 | 220 |
| PB-50/12 / 23.8 | Standard | 2 | 6.5 | 220 |

Table 8. Minimum product capacities

The potential supplier must provide an estimate of the cycle time and production capacity for each product and setting configuration listed in Table 8.

The reference products are MB2 Optim, TB20 Optim and PB-50/10/19 and they will be used to prove the production capacity according to the calculated capacities provided by the Tenderer, in a way by proving the capacity for the MB2 Optim product with the assistance and supervision of technical support of the Tenderer, while for other reference products the Ordering party will try to prove independently, with the guarantee of the Tenderer that, in case of unsuccessful proof, Tenderer will correct the deficiencies at his own expense and assist in proving the capacity.

The calculated capacity for the MB2 Optim product will be used as a line capacity criterion for the bid selection.

Obligations of the Tenderer

- Before making a commercial offer, the Tenderer must perform a detailed inspection of the plant and check all the necessary data.

- The supplier can suggest the necessary improvements in the system that could be made in order to achieve optimal results.